Weight and Balance: N783SF

|  | Weight | Arm | Moment |
| --- | --- | --- | --- |
| Empty | 1691.70 | 40.68 | 68818.356 |
| Pilot | +_________________ | 37 (34-46) | +_________________ |
| Passenger | +_________________ | 37 (34-46) | +_________________ |
| Rear Seat | +_________________ | 73 | +_________________ |
| Baggage | +_________________ | 95 | +_________________ |
| Zero Fuel Weight (6 lbs./gal) | =_________________ | CG:_______________ | =__________________ |
| Usable Fuel | +_________________ | _____ ___48_______ | +_________________ |
| Ramp Weight | =_________________ | N/A | =_________________ |
| Taxi Fuel | -_________________ | ________48_______ | -_________________ |
| Takeoff Weight | =_________________ | CG:_______________ | =_________________ |
| Fuel Burn | -_________________ | ________48_______ | -__________________ |
| Landing Weight | =_________________ | CG:_______________ | =__________________ |

Max Gross: 2550 lbs.

Useful Load: 858.3

Formulas:

- Weight X Arm= Moment

- Total Moment/ Total Weight= CG ( Center Gravity)

- Max Ramp Weight – Zero Fuel Weight = Usable Fuel Weight

- Fuel Weight/ 6= Fuel Gallons

- 100LL( Blue) Fuel Weights 6lbs./ Gal

- Oil Weights 7.5lbs/ gal