Weight and Balance: N4923L

|  | Weight | Arm | Moment |
| --- | --- | --- | --- |
| Empty | 1318.84 | 84.77 | 111801.86 |
| Pilot | +_________________ | 80.5 | +_________________ |
| Passenger | +_________________ | 80.5 | +_________________ |
| Rear Seat | +_________________ | 118.1 | +_________________ |
| Baggage | +_________________ | 142.8 | +_________________ |
| Zero Fuel Weight (6 lbs./gal) | =_________________ | CG:_______________ | =__________________ |
| Usable Fuel | +_________________ | ______95_______ | +_________________ |
| Ramp Weight | =_________________ | N/A | =_________________ |
| Taxi Fuel | -_________________ | ________95_______ | -_________________ |
| Takeoff Weight | =_________________ | CG:_______________ | =_________________ |
| Fuel Burn | -_________________ | ________95_______ | -__________________ |
| Landing Weight | =_________________ | CG:_______________ | =__________________ |

MAX Gross: 2400.00

Formulas:

- Weight X Arm= Moment

- Total Moment/ Total Weight= CG ( Center Gravity)

- Max Ramp Weight – Zero Fuel Weight = Usable Fuel Weight

- Fuel Weight/ 6= Fuel Gallons

- 100LL( Blue) Fuel Weights 6lbs./ Gal

- Oil Weights 7.5lbs/ gal