Specifier Note: Red colored text below requires you to select an option before this specification can be completed. Options in red text are bound by parenthesis. Additional BEAM information can be found on our website www.materiallogiq.com

SECTION 09 84 36

BEAM – PET Felt Acoustic Ceiling Baffle

PART 1 GENERAL

SECTION INCLUDES

BEAM – PET felt acoustic ceiling baffle.

REFERENCES

American Society for Testing and Materials (ASTM)

ASTM C1338: Standard Test Method for Determining Fungi Resistance of Insulation Materials and Facings

ASTM C423: Standard Test Method for Sound Absorption and Sound Absorption Coefficients by the Reverberation Room Method

ASTM E84: Standard Test Method for Surface Burning Characteristics of Building Materials.

ASTM E795: Standard Practices for Mounting Test Specimens During Sound Absorption Tests

ASTM E1477: Standard Test Method for Luminous Reflectance Factor of Acoustical Materials by Use of Integrating-Sphere Reflectometers

California Department of Public Health (CDPH)

CDPH Standard Method V1.2: Standard Method for the Testing and Evaluation of Volatile Organic Chemical Emissions from Indoor Sources Using Environmental Chambers

  - International Code Council (ICC)

ICC IBC: International Building Code

SUBMITTALS

Product Data: Manufacturer’s data sheet and installation instructions.

Samples: Submit, at minimum, a 4” x 4” sample for each type of specified panel type.

Test Reports: Upon request, submit certified test reports to verify specified product performance.

MAINTENANCE MATERIAL

Extra Stock Materials:

Deliver no less than two percent (2%) / (Other) of each type, color, and pattern of material.

Extra materials shall remain in the manufacturer’s original packaging and given to the building owner upon substantial completion of the work. Store extra materials per instructions as described in storage and handling requirements.

QUALITY ASSURANCE

Qualifications:

Manufacturers: Provide PET felt acoustic ceiling baffle from a single manufacturer.

Installers: Utilize an installer having demonstrated experience on projects of comparable size and complexity.

Performance Requirements:

Surface Burning Characteristics: Acoustical panels to perform as specified when tested in accordance with ASTM E84. Acoustical panel surface burning performance should comply with the International Building Code and other local building code requirements.

Acoustical Characteristics: Acoustical panels to perform as specified when tested in accordance with ASTM C423.

DELIVERY, STORAGE, AND HANDLING

Storage and Handling Requirements:

Handle products carefully to avoid damage or chipping edges.

Store products in a clean, cool, dry place, and out of direct sunlight.

Store products in a space where the ambient temperature and humidity conditions are being maintained at the levels indicated for the project when occupied for its intended use.

SITE CONDITIONS

Ambient Conditions: Maintain ambient temperature and humidity conditions at levels indicated for the project when occupied for its intended use.

Existing Conditions: Do not install ceiling panels until space is enclosed and weather proofed, wet work is completely dry, and work above ceilings is complete.

WARRANTY

Provide manufacturer’s written product warranty per Section 01 77 00 – Closeout Procedures.

PART 2 PRODUCTS

Specifier Note: Red colored text below requires you to select an option before this specification can be completed. Options in red text are bound by parenthesis. Additional PET Felt BEAM information can be found on our website www.materiallogiq.com

MANUFACTURERS

- MATERIAL LOGIQ, 123 Columbia Court N, Chaska, MN 55318.
Rep contact for spec assistance, samples, budgeting – Urban Products Group, Kevin Stasi; Phone: 856-669-9513; email kevin@urbanproductsgroup.com. Website: www.materiallogiq.com

DESCRIPTION

- Product: BEAM – PET Felt Ceiling baffle as manufactured by MATERIAL LOGIQ.

Product Options:

  - Beam Composition: 	60% Recycled PET Felt

  - Beam Thickness:	(3”) / (4”) / 5”) / (Custom)

  - Beam Height: 		(6”) / (8”) / (10”) / (Custom)

  - Beam Length:		(2’ to 10’)

  - Finish:	 		Class A Fire-Rated Colors: (White) / (Silver) / (Smoke) / (Gray) /
 			(Black) / (Beige) / (Tan) / (Brown) / (Blue) / (Purple) / (Red) /
 			(Orange) / (Lime) / (Printed Wood Grain) / (Printed Custom)

- Non-Fire Rated Colors: (Sky) / (Azure) / (Lake) / (Lavendar) / (Pink) / (Yellow) / (Tangerine) / (Ruby) / (Melon) / (Teal) / (Midnight Green)

  - Mounting Method: 	(Suspension Cable) / (Threaded Rod) / (T-Grid) / (Direct Attach)

Product Performance:

  - Surface Burning Performance

    - Fire Rating per ASTM E84: Class A.

PART 3 EXECUTION

EXAMINATION

- Verification of Conditions:

  - Inspect installation area and conditions under which work is to be performed for compliance with all manufacturers’ environmental requirements.

  - All wet work in the installation area must be complete, cured, and dry prior to installation.

  - Work above ceilings shall be complete, inspected, and accepted before ceiling work begins.

INSTALLATION

- Comply with manufacturer’s instructions and recommendations for installation of acoustical wall and ceiling panels and with industry standards.

CLEANING

- Clean surfaces of ceiling panels per manufacturer’s instructions.

- Remove and replace damaged or discolored material and material that cannot be properly cleaned.

PROTECTION

- Protect installed work from damage due to subsequent construction activity, including temperature and humidity limitations and dust control, so that the work will be without damage and deterioration at the time of acceptance by the owner.

END OF SECTION