NAKTO  ELECTRIC  BICYCLE

- LCD-M5 display Control Panel Instruction

# Size（unit：mm）

Case：ABS & Meta& glass

Front                               Side

31.8mm Rack

Front switch                                side

# Voltage and Wiring

- Voltage：DC24V、36V、48V、60V、64V

- Wiring

Front LCD side

| No. | Color | Function |
| --- | --- | --- |
| 1 | brown（VCC） | Power source |
| 2 | Green（RX） | Data receiving |
| 3 | Black（GND） | Ground |
| 4 | Orange（K） | Power switch for controller |
| 5 | white（TX） | Data Sending |

### Introduction for display marks:

All displays:

- Introduction for display marks:

1. Head light

2. Battery Level

3.Multi-function area

Total Range ODO、Single Range TRIP A、Single Range TRIP B、Voltage VOL、Currency CUR、miles left RM； Running time TM

4. Running Model

Walk: assistant walking ；Cruise：Cruiser model ；PAS：pedal assistant:0~9 levels

5. Speed display area

Unit  MPH, KM/H

6. Bike Status display

Codes ：

| Code | Meaning | Notes |
| --- | --- | --- |
| 0 | Normal |  |
| 1 | Stay |  |
| 2 | Brake |  |
| 3 | PAS sensor |  |
| 4 | 6KM/H Cruiser |  |
| 5 | Cruiser |  |
| 6 | Low voltage |  |
| 7 | Motor breakdown |  |
| 8 | Throttle error |  |
| 9 | Controller error |  |
| 10 | Signal receiving error |  |
| 11 | Signal sending error |  |
| 12 | BMS communication |  |
| 13 | Head light error |  |

| 33 | Currency abnormal |  |
| --- | --- | --- |
| 34 | Throttle Error |  |
| 35 | Motor lost one phase |  |
| 36 | Motor Magnet error |  |
| 37 | Brake Error |  |
| 30 | Communication Error |  |

7. Settings

- P01：Background lightness :

- 1dark，2 normal, 3lightest；

P02：Distant unit，

0：KM；1：MILE；

P03：Battery Voltage：

24V，36V，48V，default  36V；

P04：Sleeping time：

0，unlimited； 1-60 minutes；

P05：PAS Level:

0: 3 levels model :

Level 1: 2V，

Level 2: 3V，

Level 3: 4V；

1,: 5 levels model:

Level 1:2V(12MPH),

Level 2: 2.5V(13.1MPH)，

Level 3: 3V15.7(MPH)，

Level 4: 3.5V(16.3MPH)，

Level 5:4V(19MPH);

P06:  Wheel diameter：

Unit: inch；

P07:  No of magnet steel for speed measurement： 1-100；

P08：Speed limit：Range 0-50 km/h，50 means no limit

P09：Setting for zero/none zero start: 0：zero start ；1：none zero start；

P10：Powering Model settings:

0 : PAS(throttle disabled, output power is corresponding to PAS levels)。

1 : Throttle(PAS disabled, output power is controlled by throttle turning, top speed it corresponding to PAS level settings)。

2 : PAS & Throttle(In P9, the setting has to be 1)。

- P11：PAS sensitivity： Range 1-24；

P12：PAS starting intensity：Range 0-5；

P13：Magnet steel model for PAS:   5，8，12 three models

P14：Currency limit for controller : Range：1-20A , default 12A

P15：Controller low voltage value

P16:  ODO  reset (Push and hold up button for 5 sec)

P17：0：Cruiser disabled，1：Cruiser abled

P18：Displayed Speed ratio adjust.  range：50%~150%，

P19：0 effectiveness 0：yes，1：no

P20： Protocols

### Buttons:

The buttons can be used with press or press and hold for 3 seconds for the function to be activated.

1.

Press to increase/decrease the PAS level to control                                       the top speed

Press and hold both buttons to enter/exit to settings.

2. /Press and hold “M” to turn on and off LCD display and Power

3. + Push and hold to change the display of speed unit

### Settings

- Set/Cancel 4MPH Cruiser, headlight, ODO reset

During the stop of the bike, Push and hold ，To enter 4MPH Cruisermodel. Release the  button to stop

Press and hold for 2 secs  to Switch on/off the headlight.

On  P16 status, Press and hold  for 5 Sec，ODO reset。

- Turn on/ off LCD screen

Press and Hold “M” to Turn on/off the LCD creen.

- Switch between Multi-functional displays

Click “M” to switch between function displays

- Press and hold +  to enter setting:

Use ，or to increase or decrease the value, or push and hold ，or  To change the value faster.

Press “M” to switch between settings；

- Press and hold + to exit settings and save value，or you can leave it for 8secs and it will exit and save values.