# ___________________________________

# EDGEWATER CUSTOM GOLF CARTS

4890 Edgewater Dr

Orlando Fl  32804

Phone - 407-295-3279

Edgewatercustom@Hotmail.com

EDGEWATERCUSTOMGOLF.COM

36V-48V Voltage Reducer 10 Amp Max Load

Installation Instructions For LV36/48-12

Choose the location on the metal portion of your golf car chassis for installation with good ventilation.  Place voltage reducer on chassis and mark holes.  Drill holes in frame and bolt into place.

With a voltage tester locate where you have 36Volts or 48Volts on your batteries, usually a BLACK wire from the controller to a battery on one side of the cart and a RED wire to a battery on the other side of the cart.

CAUTION

If your cart has a Run Tow switch, flip to tow before removing a battery cable to prevent sparking at the battery terminal!         “Always use eye protection”

IMPUT:  at the input side of the reducer connect these wires to the 36 or 48 volt points on the golf cart. Connect the BLACK wire to the negative battery source and connect the ORANGE wire to the positive battery source.

Remove and the Negative side of the battery pack cable first to prevent sparks. Install the ORANGE input wire to the positive side of the battery pack first. Next install the BLACK wire to the negative side of the battery pack and re install the negative battery cable.

OUTPUT:  This is the output 12 volt DC side of the reducer.  Connect the BLACK wire to your ground block along with the negative side of all electrical components installed on cart and the RED wire to the positive side of all electrical components installed.

You will now draw your 12V power from all of the battery pack. Do not exceed a maximum of 10 amp operating current with the items you plan to operate at one time. Normally lights, horn and a standard radio will be under 10 Amp draw.