EZ-GO DIY BRAKE LIGHT SWITCH KIT WITH TIME OUT MODULE

Caution

For carts with a RUN/TOW switch turn switch to TOW before removing battery cables, all others remove the ground wire first to prevent arcing. Always wear eye protection.

- Connect the fused (+) 12 Volt Power to the brake light switch

- Connect Positive (+) 12 Volt Power from brake light switch to Terminals #2

- Connect Negative (-) to Terminal #3

- Connect Brake Lights to Terminal #1

- Mount the switch and bracket under the front of the cart above the brake pedal arm. Mount bracket by drilling holes in floor using bolts and nuts to secure.

- Switch should be depressed with brake off.

When the brake is applied switch will supply power to the time out switch allowing current to flow to the brake lights.

- Mount the time out switch.

- Connect the 12 volt positive wire (with fuse holder) to the battery.

- Connect the 12 volt negative to the battery.

- Reconnect the battery cable or flip Tow/Run switch.

When the parking brake is applied the brake lights will turn off after 30 seconds.

INCLUDES

- Switch

- Timer

- Bracket

- Fuse Holder

- Fuse

- Terminals

# ____________________________________________________________

# EDGEWATER CUSTOM GOLF CARTS

4890 Edgewater Dr

Orlando Fl. 32804

407-295-3279

Edgewatercustom@hotmail .com

__2

TIMER

__3               1__

Brake Light

Switch

FUSE

Battery #2

_                  +

Battery #1

+                   _

EZ-GO 36 Volt Brake Light Wiring Diagram

For Gas Carts connect to the 12 Volt Battery

Tail Light

POSITIVE

Tail Light

NEGATIVE