EZ-GO BRAKE LIGHT SWITCH KIT WITH TIME OUT MODULE

Caution

For carts with a RUN/TOW switch turn switch to TOW before removing battery cables, all others remove the ground wire first to prevent arcing. Always wear eye protection.

- Mount the switch and bracket under the front of the cart above the brake pedal arm. Mount bracket with screws supplied or some models may require drilling holes in floor using bolts and nuts to secure.

- Switch should be depressed with brake off.

When the brake is applied switch will supply power to the time out switch allowing current to flow to the brake lights.

- Mount the time out switch.

- Connect the 12 volt positive wire (with fuse holder) to the battery.

- Connect the 12 volt negative to the battery.

- Connect the long wire with the male spade terminals to the brake lights.

When the parking brake is applied the lights will turn off after 30 seconds.