YAMAHA DIY BRAKE LIGHT SWITCH KIT WITH TIME OUT MODULE

Caution

For carts with a RUN/TOW switch turn switch to TOW before removing battery cables, all others remove the ground wire first to prevent arcing. Always wear eye protection.

- Connect the fused (+) 12 Volt Power to the brake light switch

- Connect Positive (+) 12 Volt Power from brake light switch to Terminals #2 & #4

- Connect Negative (-) to Terminal #3

- Connect Brake Lights to Terminal #1

- Mount the switch and bracket under the cart by the brake cable adjusting nut.

- Mount the small bracket to the brake cable with the jam nuts on the brake cable.

- Switch will be pulled when the brakes are applied.

When the brake is applied switch will supply power to the time out switch allowing current to flow to the brake lights.

- Mount the time out switch.

- Connect the 12 volt positive wire (with fuse holder) to the battery.

- Connect the 12 volt negative to the battery.

- Reconnect the battery cable or flip Tow/Run switch.

When the parking brake is applied the brake lights will turn off after 30 seconds.

INCLUDES

- Switch

- Timer

- Switch Bracket

- Small Bracket

- Fuse Holder

- Fuse

- Terminals