# Vuzix M Series Quick Start Guide

M400

M4000

# Contents

- Prepare

  - Creating a Mobile Hotspot

    - Android

    - Apple

  - Connecting to Wi-Fi

    - Android

    - Apple

- Voice Commands

- Help Lightning App

  - Making a Video Call

- Prepare

  - Creating a Mobile Hotspot

    - Android

- Open the Settings on your mobile device.

- Select Connections.

- Select Mobile Hotspot and Tethering.

- Select Mobile Hotspot.

- Select the Network name field and enter in what you would like your mobile hotspot to be known as, e.g. John Smith’s Wireless Hotspot.

- Enter in what you would like the password to be into the Password field.

- Press the Save button.

- At the top of the screen press where it says Off to turn the mobile hotspot On.

    - Apple

- Open the Settings on your mobile device.

- Select Cellular.

- Select Set Up Personal Hotspot.

- Follow the onscreen instructions.

  - Connecting to Wi-Fi

    - Android

On your mobile device

- Open the Settings on your mobile device.

- Select Connections.

- Select Mobile Hotspot and Tethering.

- Select Mobile Hotspot.

- At the bottom of the screen press QR code to generate a QR code.

On the Vuzix Blade

- Navigate to and open Scanner.  The QR scanner will open on your Vuzix Smart Glasses.

- Move your head or phone, so that the QR code on your phone appears on the Vuzix Smart Glasses screen and hold it there until it has finished reading the QR code.

    - Apple

On your mobile device

- Open your web browser on your mobile device.

- Enter https://www.vuzix.com/pages/wifi-qr-generator into the address bar in your web browser.

- Enter in the WiFi hotspot name that you created on your mobile device into WIFI NAME.

- Enter in the WiFi hotspot password into PASSWORD.

- Press the GENERATE button to generate a QR code.

On the Vuzix Blade

- Navigate to and open Scanner.  The QR scanner will open on your Vuzix Smart Glasses.

- Move your head or phone, so that the QR code on your phone appears on the Vuzix Smart Glasses screen and hold it there until it has finished reading the QR code.

- Voice Commands

You can navigate your Vuzix smart glasses using voice commands.  To start using voice commands on your smart glasses you say “Hello Vuzix” and then proceed to give voice commands to open apps and make video calls from your smart glasses.

Below is a list of voice commands that the Vuzix smart glasses recognise.

| Voice Command | Action |
| --- | --- |
| Hello vuzix | Activates the listener |
| Go left | Basic directional navigation |
| Move left | Basic directional navigation |
| Go right | Basic directional navigation |
| Move right | Basic directional navigation |
| Go up | Basic directional navigation |
| Move up | Basic directional navigation |
| Go down | Basic directional navigation |
| Move down | Basic directional navigation |
| Pick this | Activates the item that currently has focus |
| Select this | Activates the item that currently has focus |
| Confirm | Activates the item that currently has focus |
| Okay | Activates the item that currently has focus |
| Open | Activates the item that currently has focus |
| Scroll down | Basic directional navigation |
| Scroll left | Basic directional navigation |
| Scroll right | Basic directional navigation |
| Scroll up | Basic directional navigation |
| Cancel | Navigates backward in the history stack |
| Close | Navigates backward in the history stack |
| Go back | Navigates backward in the history stack |
| Done | Navigates backward in the history stack |
| Go home | Triggers the Home action |
| Quit | Triggers the Home action |
| Exit | Triggers the Home action |
| Stop | Stops the scrolling action |
| Show menu | Brings up context menu for current UI screen |
| Next | Advances to the next item in an ordered collection of items |
| Previous | Goes backward by one item in an ordered collection of items |
| Go forward | Navigates forward in the history stack |
| Page up | Navigates up one page |
| Page down | Navigates down one page |
| Volume up | Increases volume by 5 |
| Volume down | Decreases volume by 5 |
| Speech settings | Opens Speech Settings menu |
| Speech commands | Opens the current Speech Command list |
| Command list | Opens the current Speech Command list |
| Start recording | Opens Camera app to the record function (to begin recording, use the 'Select This' or 'Pick This' command) |
| Take a picture | Opens Camera app to the picture function (to take a picture, use the 'Select This' or 'Pick This' command) |
| View notifications | Opens the Notification Manager |
| Open notifications | Opens the Notification Manager |
| Clear notifications | Clears all Notifications |
| Voice off | Stops the listener |

- Help Lightning App

  - Making a Video Call

- Say “Hello Vuzix” to start using voice commands.®

- Say “Go to scanner” to open the scanner.

- Scan the QR code of the person that you wish to video call.

- Say “Open” for the “Open Browser” button. You may have to say “Open” a few times for the QR code to open the Help Lightning app, if you have not selected for it to automatically open the Help Lightning app.

- Say “Go right” to highlight the “Always” button.  Saying “Open” on the “Just Once” buttons means that you will have to do this every time you scan a QR code to call someone.

- Say “Select” to select the “Always” button.

- Once the Help Lightning app opens on your smart glasses, the video call is automatically made to the person.