Table of Contents

- Installing the HALO Smart Hub Mini
 How to safely mount and wire the device into the wall using mains power.

- Initial Setup (Boot Guide)
 Steps to power on the device, choose a language, connect to Wi-Fi, and pair with the app.

- Navigating the Interface
 An overview of what you see on the screen and how to access different functions.

- Controlling Devices
 How to find, view, and control smart devices from the panel.

- Scene Control (Tap-to-Run)
 How to run automated actions with a single tap using pre-set scenes.

- Settings & Customisation
 Where to find and change key settings like Wi-Fi, display, theme, and integrations.

- Troubleshooting
 Basic solutions for common problems like connection issues or setup failures.

## 1. Installing the HALO Smart Hub Mini

Electrical installation should be carried out by a qualified professional.

### Step 1: Turn Off the Power

- Locate your consumer unit (fuse box).

- Turn off the mains circuit breaker that controls the installation area.

### Step 2: Prepare the Wall

- Ensure the wall box is clean and securely fitted.

- If not pre-installed, fix the mounting bracket to the back box using the included screws.

### Step 3: Wiring

- Use a voltage tester to ensure there is no live current.

- Connect the wires as follows:

  - L (Live) – Brown wire

  - N (Neutral) – Blue wire

- Double-check connections are firm and properly secured.

### Step 4: Mount the Panel

- Align the Smart Hub with the wall bracket.

- Press the hub firmly until it clicks into place.

### Step 5: Restore Power

- Return to the consumer unit and switch the power back on.

- The HALO Smart Hub Mini should power up automatically.

## 2. Initial Setup (Boot Guide)

### Power On

- The screen will light up, and you’ll see a welcome message.

### Select Language

- Choose your preferred language (English UK recommended).

### Connect to Wi-Fi

- Tap “Settings” > “Network”.

- Select your 2.4GHz home Wi-Fi network.

- Enter your Wi-Fi password carefully and confirm.

### Pairing with App

- Open the VIVOE app.

- On the app home screen, tap the “+” in the top right corner.

- Select “Scan” and scan the QR code shown on the panel screen.

- Wait a few seconds for the control panel to sync with your app devices and rooms.

### Complete Setup

- Once linked, the HALO Smart Hub Mini will sync with your VIVOE account and any connected devices.

## 3. Navigating the Interface

### Home Screen

The home screen layout includes:

- Top Bar: Shows time, date, local weather, and Wi-Fi signal.

- Room Shortcuts: Tap to view devices grouped by room.

- Scene Shortcuts: Tap to trigger your custom automations.

- Menu Icon: Access settings, pairing, themes, and system options.

Note: Room, scene, and tap-to-run shortcuts need to be programmed in the VIVOE app and will automatically show on the panel.

## 4. Controlling Devices

### Accessing Devices

- Swipe right from the home screen to access All Devices.

- Devices added via the app will automatically appear.

### Controlling Devices

- Tap the power icon on a device card to toggle it on/off.

- Tap the device card to open the Advanced Control Page.

- Tap All Devices (top left) to filter by room.

### Customising the Display

Long-press a device card to:

- Reorder the display

- Hide devices (can be restored later)

## 5. Scene Control (Tap-to-Run)

### Accessing Scenes

- Swipe left from the home screen to view Tap-to-Run scenes.

### Running a Scene

- Tap a scene card to trigger a pre-set group of actions (e.g. "Movie Time").

### Scene Options

- Tap Tap-to-Run (top left) to switch between lighting and other scenes.

- Long-press to hide, reorder, or restore scenes.

## 6. Settings & Customisation

### Accessing Settings

- Swipe down from the top and select the gear symbol.

### Key Settings

- Wi-Fi Network: Change or reconnect.

- Display Brightness: Auto or manual brightness.

- Theme: Choose dark or light mode, or a colourful layout.

- Voice Assistant Integration: Connect to Alexa or Google Assistant.

- Firmware Update: Keep your device up to date for best performance.

7. Troubleshooting

| Problem | Solution |
| --- | --- |
| Screen not powering on | Check mains connection and breaker. Ensure wiring is correct. |
| Cannot connect to Wi-Fi | Use 2.4GHz only. Check password and router proximity. |
| Device pairing fails | Reset the smart device and retry. Keep within 5 metres of the hub. |
| Scenes not running | Verify conditions and device connectivity. Check time zone settings. |