LEVELING BOLTS

THUMB SCREW

PISTON

CYLINDER

PUMP

Basic instructions describing the operation of our stone chip repair system.

- Place holding structure on glass surface with opening for injector directly over the impact of the break.  Do not use Vaseline on suction cup of pump-style holder.

- Holding injector upside down, remove grey cap, loosen locking screw on injector one half turn, pull plunger down until it stops

- Using seal removal tool (plastic stick) remove white injector seal

- Load injector with 3 to 4 drops of 20cps or 40cps resin

- Replace seal and carefully push plunger up until resin is almost flush with top of injector seal then turn locking screw tightly

- Load injector into holding structure (injector must be at right angle as it is dual threaded)

- Turn injector until it touches the glass, then turn one full turn further

- Loosen lock screw half turn only and while securely holding the injector, pull plunger up until it stops then turn lock screw tightly.  This is the vacuum cycle, leave it on for 60 seconds

- Holding plunger, loosen lock screw allow it to slowly lower until it stops, then slightly lift injector while applying pressure to plunger

- Looking at bottom of injector from the side, you will see the injector seal bloat out of the injector indicating pressure. When the seal is about half way out, tighten the lock screw.  This is the pressure cycle

- As the resin fills the break you may employ techniques such as flexing and tapping to fill the tight spots

- For breaks with complex damage at the impact area such as combination breaks, pockets of air can be removed by “cycling” the system from vacuum to pressure and back several times as needed

- When break is completely filled, cure repair while resin is still under pressure using quick cure lamp for 60 seconds

- After curing, release pressure by loosening the lock screw one half turn, then release suction cup on holding structure and move assembly away from the repair area

- Place one drop of pit fill resin on pit area, roll tab on (to remove air bubbles from pit area) and cure

- Remove injector from holding structure and place cap on it, remove holding structure BY PRESSING ON TABS LOCATED ON SUCTION CUP. Place both back into kit

- Remove tab from pit by lifting at one edge and “peeling it back”, scrape excess resin with razor blade at 90 degree angle

- Finally, shake the polish/sealant and apply one drop to pit, polish to a glossy finish with a paper towel and repair is done

CLEANING INSTRUCTIONS

- Remove cap and seal, pour out unused resin

- Loose lock screw several turns and pull plunger and collar out of injector cylinder

- Remove seal from plunger

- Place all components into cleaning jar filled with regular rubbing alcohol then remove

- Wipe all components and reassemble

- Lubricate the plunger seal with one full drop of stone chip resin, then insert it into the injector

- Replace cap and injector is ready for use