COSHH Risk Assessment – Talon Soft XT

# 1. Substance Information

Product Name: Talon Soft XT

Type: Rodenticide (soft block bait)

Active Ingredient: Brodifacoum 0.0025–0.003% w/w

Manufacturer: Syngenta UK Ltd, CPC4 Capital Park, Fulbourn, Cambridge CB21 5XE

Emergency Contact: +44 (0)1484 538444 (24 h)

Product Registration No.: GB-2017-1115-3-0002

# 2. Intended Use

For professional use in the control of rats and mice indoors, and around buildings.

Applied as a soft bait within tamper-resistant bait stations, clearly labelled and secured against non-target access.

Use limited to trained pest-control technicians in compliance with the UK Rodenticide Stewardship Regime (CRRU).

# 3. Hazard Identification

Classification (UK CLP):

• 	H373 – May cause damage to organs (blood) through prolonged or repeated exposure Signal Word: Warning

Pictograms: GHS08 (Health hazard)

Contains: Brodifacoum (ISO), 1,2-benzisothiazol-3(2H)-one (may cause allergic reaction)

# 4. Routes of Exposure

- Ingestion by accidental hand-to-mouth contact

- Skin contact during bait placement or station servicing

- Inhalation of dust when handling contaminated packaging

- Secondary exposure via contaminated surfaces or PPE

# 5. Control Measures (HSE / BPCA Hierarchy)

Elimination/Substitution: No safer alternative of equivalent efficacy.

Engineering Controls: Use only in enclosed bait boxes designed to prevent non-target access.

Administrative Controls:

- Record site, location, and bait quantities.

- Remove bait at treatment end and document in treatment log.

- Prevent access by public, pets, and wildlife. Personal Protective Equipment (PPE):

- Disposable coveralls (Type 5/6)

- Nitrile gloves (EN 374)

- Safety glasses (EN 166) when handling bulk bait

- Respiratory protection: not normally required; use P2 mask if dusty conditions occur

- Inhalation: Move to fresh air; seek medical advice if unwell.

- Skin Contact: Wash immediately with soap and water. Remove contaminated clothing.

- Eye Contact: Rinse cautiously with water for at least 15 minutes. Seek medical attention if irritation occurs.

- Ingestion: Seek immediate medical help and show product label or SDS. Do not induce vomiting. Antidote: Vitamin K₁ (phytomenadione).

Medical Advice: Monitor prothrombin time and coagulation factors for up to 48 hours post-exposure.

# 7. Spillage & Emergency Procedures

- Contain and collect spilled bait using disposable gloves.

- Avoid generating dust.

- Prevent contamination of drains and watercourses.

- Dispose of waste through licensed hazardous-waste contractor.

# 8. Storage & Handling

- Store in original packaging, in a cool, dry, locked pesticide store.

- Keep away from food, feed, and water supplies.

- Store only in areas compliant with BPCA / HSE COSHH guidance and CRRU UK standards.

# 9. Environmental Protection

- Extremely toxic to wildlife and aquatic organisms.

- Prevent release into watercourses or external soil.

- Retrieve all uneaten bait at treatment end.

- Use secure bait stations and follow CRRU “Think Wildlife” Code of Best Practice.

# 10. Disposal

- Collect bait residues and empty packaging.

- Dispose of via licensed waste contractor as hazardous waste (EWC 20 01 19*).

- Do not burn or bury containers.

# 11. Health Surveillance

- Maintain treatment and exposure logs for all operatives.

- Monitor for symptoms of anticoagulant poisoning (bruising, bleeding gums, blood in urine/faeces).

- Retain COSHH records for minimum 5 years.

# 12. Assessment Review & Sign-Off

Technician: Kurtis Heath

Date: October 2025

Next Review Date: October 2026