BOWMAN'S PEST MANAGEMENT LTD
COSHH RISK ASSESSMENT – IMIDASECT ANTS

1. Substance Information

Product Name: IMIDASECT ANTS

Type: Ant Gel Bait (Biocide)

Active Ingredient:
• Imidacloprid 0.01% Gel

Product Code:
SHA 3200 B

Manufacturer:
Sharda Cropchem Ltd.
Mumbai, India

Emergency Contact:
+91 22 6678 2800
CARECHEM UK (24 Hour): +44 (0)1865 407333

2. Intended Use

Professional ant control gel bait for use against ants.
For application by trained pest control technicians.

3. Hazard Identification

Signal Word: NONE REQUIRED

Hazard Classification:
Product not classified under CLP.

Active Ingredient Hazards:
• H302 – Harmful if swallowed
• H400 – Very toxic to aquatic life
• H410 – Very toxic to aquatic life with long lasting effects

4. Routes of Exposure

• Inhalation
• Skin Contact
• Eye Contact
• Ingestion

5. Control Measures

Engineering Controls:
• Avoid contact with skin and eyes.
• Wash hands after use.
• Do not eat, drink or smoke during application.

PPE:
• Chemical resistant gloves (EN374)
• Eye protection (EN166)
• Protective clothing where necessary
• P2/P3 respirator if exposure to particles is possible

6. First Aid Measures

Inhalation:
Move casualty to fresh air. Seek medical attention if required.

Skin Contact:
Remove contaminated clothing and wash thoroughly with water.

Eye Contact:
Wash eyes with water for at least 20 minutes and seek medical advice.

Ingestion:
Rinse mouth and immediately contact a doctor or poison centre.

7. Spill and Emergency Procedures

• Wear PPE.
• Absorb using sand, sawdust or inert absorbent material.
• Prevent contamination of drains and watercourses.
• Clean contaminated area with water.

8. Storage and Handling

• Store in original container.
• Store in a cool, dry and well ventilated area.
• Protect from direct sunlight.
• Avoid contamination of food and feed.
• Keep out of reach of unauthorised persons.

9. Environmental Protection

• Prevent contamination of drains, ponds and watercourses.
• Avoid release into the environment.
• Notify authorities if product enters public water systems.

10. Disposal

Collect waste into suitable labelled containers and dispose of in accordance with local regulations.

11. Health Surveillance

Monitor for:
• Skin irritation
• Eye irritation
• Symptoms following accidental ingestion

12. Assessment Review & Sign-Off

Technician: Kurtis Heath
Assessment Date: 07/06/2026
Review Date: 07/06/2027

Signature: ____________________