BOWMAN'S PEST MANAGEMENT LTD
COSHH RISK ASSESSMENT – ERADISECT FLYING & CRAWLING INSECT KILLER

1. Substance Information

Product Name: Eradisect Flying & Crawling Insect Killer

Type: Aerosol Insecticide

Active Ingredients:
• Cypermethrin (<1%)
• Piperonyl Butoxide (1–5%)
• Chrysanthemum Cinerariaefolium Extract (<1%)

Manufacturer:
PelGar International Ltd
Unit 13 Newman Lane
Alton
Hampshire
GU34 2QR

Emergency Contact:
+44 (0)1420 80744
NHS 111

2. Intended Use

Professional and domestic insecticide for the control of flying and crawling insects.
Applied as a ready-to-use aerosol spray.

3. Hazard Identification

Signal Word: DANGER

Hazard Statements:
• H222 – Extremely flammable aerosol
• H229 – Pressurised container: May burst if heated
• H315 – Causes skin irritation
• H410 – Very toxic to aquatic life with long lasting effects
• EUH208 – May produce an allergic reaction

4. Routes of Exposure

• Inhalation of spray mist or vapours
• Skin contact
• Eye contact
• Accidental ingestion

5. Control Measures

Engineering Controls:
• Ensure adequate ventilation.
• Avoid spraying near ignition sources.
• Prevent environmental contamination.

PPE:
• Protective gloves
• Protective clothing/coveralls
• Safety glasses or goggles
• Respiratory protection where ventilation is inadequate
• Safety footwear

6. First Aid Measures

Inhalation:
Move casualty to fresh air and keep comfortable for breathing.

Skin Contact:
Wash thoroughly with soap and water.

Eye Contact:
Rinse cautiously with clean water.

Ingestion:
Rinse mouth. Do not induce vomiting. Seek medical advice immediately.

7. Spill and Emergency Procedures

• Eliminate ignition sources.
• Ventilate area.
• Absorb with sand, earth or vermiculite.
• Prevent entry into drains and watercourses.

8. Storage and Handling

• Store in original container.
• Keep locked away when not in use.
• Protect from direct sunlight.
• Do not expose to temperatures above 50°C.
• Store upright in a cool, ventilated area.

9. Environmental Protection

• Very toxic to aquatic life with long lasting effects.
• Avoid contamination of drains, ponds, rivers and watercourses.

10. Disposal

Dispose of contents and containers through licensed hazardous waste disposal routes.
EWC Code: 20 01 19*

11. Health Surveillance

Monitor for:
• Skin irritation
• Allergic reactions
• Respiratory irritation
• Dizziness or headaches

12. Assessment Review & Sign-Off

Technician: Kurtis Heath
Assessment Date: 07/06/2026
Review Date: 07/06/2027