# Bowman's Pest Management Ltd

# COSHH RISK ASSESSMENT

## Rodilon® Soft Block

## 1. Substance Information

Product Name: Rodilon® Soft Block

Product Type: Ready-to-use anticoagulant rodenticide bait (RB)

Intended Use: Professional rodenticide for the control of rats and mice in domestic, commercial, industrial and agricultural premises.

Manufacturer: 2022 Environmental Science (Envu)

## 2. Hazard Identification

Although the product is supplied as a ready-to-use bait and is not classified as hazardous under the current CLP classification, accidental ingestion can result in serious anticoagulant poisoning. Exposure should therefore be minimised through safe working practices.

Potential Health Hazards

- Harmful if swallowed.

- May cause internal bleeding if significant quantities are ingested.

- Minor irritation may occur following prolonged skin or eye contact.

- Hazardous to non-target animals if consumed.

## 3. Routes of Exposure

- Accidental ingestion.

- Skin contact during handling.

- Eye contact from contaminated hands.

- Secondary exposure through poor hygiene practices.

## 4. Persons at Risk

- Pest Control Technicians.

- Customers.

- Occupants.

- Children.

- Pets.

- Members of the public.

- Non-target wildlife.

## 5. Control Measures

### Engineering Controls

- Use tamper-resistant bait stations where appropriate.

- Secure bait stations to prevent movement.

- Ensure adequate site signage where necessary.

### Safe Working Procedures

- Read and follow the product label before use.

- Carry out a site-specific COSHH assessment before treatment.

- Prevent access by children, pets and non-target wildlife.

- Wash hands thoroughly after handling bait.

- Do not eat, drink or smoke whilst handling the product.

- Remove all visible rodent carcasses promptly.

- Record bait locations and treatment details.

### Personal Protective Equipment (PPE)

- Disposable nitrile gloves.

- Safety glasses where contamination is possible.

- Suitable work clothing.

- Waterproof footwear where required.

## 6. First Aid Measures

Inhalation

Move to fresh air if symptoms occur.

Skin Contact

Wash thoroughly with soap and water.

Eye Contact

Flush immediately with clean water for several minutes.

Seek medical advice if irritation persists.

Ingestion

Do not induce vomiting.

Rinse mouth thoroughly.

Seek immediate medical attention.

Symptoms following significant ingestion may include:

- Nose bleeds.

- Bleeding gums.

- Bruising.

- Blood in urine.

- Blood in stools.

## 7. Spill Procedure

- Wear suitable PPE.

- Collect spilled bait immediately.

- Prevent contamination of drains and watercourses.

- Dispose of contaminated materials safely.

- Prevent access by children, pets and wildlife.

## 8. Storage

- Store in the original labelled container.

- Keep securely locked.

- Store in a cool, dry, well-ventilated location.

- Keep away from food, drink and animal feed.

- Keep out of reach of children and unauthorised persons.

## 9. Environmental Precautions

- Avoid contamination of drains and watercourses.

- Dispose of rodent carcasses responsibly.

- Remove unused bait after treatment where appropriate.

- Prevent access by non-target wildlife.

## 10. Disposal

- Dispose of unused bait in accordance with local authority regulations.

- Do not dispose of bait into drains.

- Empty containers must not be reused.

- Dispose of contaminated waste through an authorised contractor where required.

## 11. Health Surveillance

Routine health surveillance is not normally required. Technicians should report any signs of skin irritation or suspected accidental exposure immediately. Any accidental ingestion must be referred for urgent medical assessment.

## 12. COSHH Assessment Details

Company: Bowman's Pest Management Ltd

Assessor: Kurtis Heath

Assessment Date: October 2025

Review Date: October 2026

Assessment Outcome: Suitable for professional use when handled in accordance with the manufacturer's instructions, this COSHH assessment and current UK legislation.

### Assessor Signature

Kurtis Heath

Date Signed: October 2025