San-ei Kagaku

Via Fill Pastes from San‐ei Kagaku

Meet the Most Demanding Requirements

Minimal Z‐Axis expansion under stress allows intricate build‐up designs

Excellent paste hardness after cure enables flat pads after sanding

¾ Packaged in 1Kg containers or in 0.5 Kg tubes.

¾ Tubes are compatible with standard automated via filling machines

¾ Packaging of paste is void‐free in the tubes; excellent no‐void filling results

¾ Improved pot life at room temperature of up to 7 days

¾ Pastes with High Tg and Low CTE above Tg (see data on reverse side)

¾ Excellent flow through high aspect ratio holes

¾ No chemical attack during desmearing

¾ NASA outgas compliant pastes

¾ Halogen free and RoHS Compliant

¾ UL V‐0 approved

For more information please contact all4‐PCB at 1 866 734 9403

| Property |  | Unit/Condition |  | IR‐6P | IR‐10FE |
| --- | --- | --- | --- | --- | --- |
| Viscosity |  | Pa.s/25°C |  | 35±5* | 40±5* |
| Shelf Life |  | ≤5°C |  | 3 months | 3 months |
| Pot Life |  | 25°C |  | 7 days | 7 days |
| Tg (TMA) |  | °C |  | 168 | 158 |
| CTE | β1 | ppm/°C |  | 39 | 38 |
|  | β2 | ppm/°C |  | 105 | 89 |
| Young’s Modulus (DMA) | 30°C | M Pa |  | 5200 | 8060 |
|  | 100°C |  |  | 4300 | 6030 |
|  | 150°C |  |  | 2600 | 4030 |
|  | 200°C |  |  | 870 | 2110 |
|  | 250°C |  |  | 660 | 1620 |
| Water Absorption | JIS C6481 | % |  | 0.16 | 0.18 |
| Filler Size | Avg. | µm |  | 2‐3 | 2‐3 |
|  | Max. |  |  | 15.0 | 15.0 |
| Solids |  | % |  | 100 | 100 |
| Outgas Testing (ASTM E 595) | TML | % |  | 0.38 | 0.26 |
|  | CVCM | % |  | 0.062 | 0.003 |
|  | WVR | % |  | 0.23 | 0.17 |
| Halogen Free |  |  |  | Yes | Yes |
| UL Approval |  |  |  | 94V‐0 | 94V‐0 |
| Cu Peel Strength |  | kg/cm |  | 0.67 | 0.89 |
| Cure Profile (surface temperature) |  | °C x minutes |  | 140x20 | 110x60 + 150x30 |

San‐ei Kagaku PHP‐900 Series

Very Notable Features:

¾ Very fast process cycle time possible with IR‐6P (20‐minute cure)

¾ Best in class overall end‐user performance with IR‐10F products

o Very high Cu peel strength

o Low CTE above Tg

o Very tough performance at elevated temperatures as noted in the Young’s Modulus

data