article

Next-Generation High-Speed Materials: An Updated List

by Tom Doslak

Streamline CirCuitS

The  evolution of high-speed PCB materials moves just  as quickly as the  circuit that passes through them. It is no surprise that sometimes a very busy  product developer might be confused by, or simply be unaware of, the  newest ad- vancements in  cutting-edge substrates. Because of this, I would like to reach out  to the  product development community to help educate them and make them aware  of the  latest in  PCB ma- terials.

Designers must develop products with more functionality, at  faster  speeds, in  smaller pack- ages.  This  has  been going on  for  generations, but  there has  been a major push by chip manu- facturers. These  chip packages (BGAs and now microBGAs)  allow    tremendous  functionality with pin   counts in  excess  of  7,000 points on

.25   mm  pitch  microBGAs, while demanding extreme circuit speeds in  excess  of 50 gigahertz and tighter tolerances throughout the  PCB.

This  seems  like  an  impossible task.  But  for- ward-thinking PCB and material manufacturers continue to reinvest in advanced equipment for

critical process consistency and advanced capa- bilities. We can  now support today and tomor- row’s next-generation high-speed requirements. This  leaves  a product designer or engineer with two  critical decisions: Choosing the  right  PCB vendor, and selecting the  product material that optimizes both performance and cost.

Analog or Digital?

Two  basic  types of  circuits fall  in  the  cate- gory  of high-speed and low-loss materials. The first is high-frequency analog (RF/microwave), and the  second is high-speed digital. The RF mi- crowave circuits need to have process precision for  tight tolerance on  their signals. These  ma- terial types have been developed over  time for critical signal loss and high frequency. There are two  types of critical signal loss—one is due  to reflection and other is loss  of circuit energy by way  of the  dielectric material. Signal  reflection is  caused by  controlled impedance variations or   mismatches.  These   impedance  variations are  caused mostly by the  PCB vendor’s process consistency and capabilities, along with mate- rial  functionality that can  vary  with change in temperature and frequency.

article

NExT-GENErATIoN hIGh-SPEED mATErIAlS: AN uPDATED lIST continues

The  high-speed digital products are  usually very  complex, high-layer count, high-density (sub  3 mil  traces) performing at  very  high fre- quencies and speeds. These  products also  have demanding material parameters, along with ad- vance physical requirements as well.  The  over- all  industry still  sees  these two  basic  types of stand-alone circuit requirements, but  the  trend in the  last couple years  has  been towards all-in- one  next-generation materials.

These advanced materials now focus on both material  parameters   (Dk,Df,CTE,Tg,Td, Z-axis) and physical structures (thin cores  and consis-

tent  variation) with high-performance blends for manufacturing advantages and overall costs savings. These  new  materials help product de- velopers in  today’s products with more robust thermal and electrical properties.

We deal  with these high-speed materials ev- ery day, and we know what substrate works,  and with which applications. We do not sell materi- als of any  kind, so I believe we can  provide an unbiased description of each material.

We  have created this   updated list  of  high- speed materials. We consider these to be the  best next-generation, high-speed PCB materials available  today.  All  of   these  materials  have been tested, proven, and are ready for pro- duction. For  easy  reference, each material line item features a product name, vendor, summa- ry, and data sheet.

This    list    will   be   continuously  updated. I hope it  proves   to  be  a  valuable resource for PCB   designers,  engineers   and    fabricators.   I value    your    feedback.  Let   me   know  if   you have  any  questions or comments about these materials.  PCBDESIGN

tom Doslak is vice president of sales and marketing for Streamline Circuits and  has more  than  22 years of PCB sales experience. to contact Doslak,  click here.

Video Interview

Kozio: Design Verification and Test

by Real Time with... PCBDesign007

Bob Potock, marketing VP of Kozio, explains Kozio’s software-based solution  for comprehensive testing and verification of PCBs.

CliCk

To View

realtimewith.com