REQUEST FOR QUOTATION

THE COMMON AT SINNOTT FARM

PO. BOX 681

# BLOOMFIELD, CT 06002

The Common at Sinnott Farm is a private community located in the northwest section of Bloomfield, off Terry Plains Road.  There are currently two “private ways” which we feel require attention.  They are Quail Run Lane and Spice Bush Lane.

The Board of Directors of the Common at Sinnott Farm respectfully requests a Quotation for the following:

- Inspect for pothole, catch basin repairs, or areas of crazing.  Areas of concern should be cut out, existing asphalt removed, and base replaced if necessary.  Areas of concern are to be repaved with a minimum of two (2) inches of bituminous concrete binder, and a minimum of one (1) inch of bituminous concrete top.

- Inspect for any cracks within pavement.  Cracks determined to be serious in nature must be blown out then sealed using a sealant that is flexible in both hot and cold weather and does not track on shoes or stick to tires.  Sealant that meets Specification SS-S-1401C ASTM D 3405, or it’s equivalent, is recommended.

- Seal coat so as to protect the road form further oxidation, oil and gas spills and the elements.  The seal coat must meet with Federal Specification RP355E ASTM 3320, or it’s equivalent, and includes sand (4 to 6 pounds with each gallon of emulsion).

- The contractor shall provide to the Board of Directors of the Common at Sinnott Farm a written report of their inspection results.

- Other contractor recommendations.  Please be specific.

- The contractor shall be responsible for coordinating with any and all utilities, to include but limited to Metropolitan District Company, for work which may impact the particular utilities.

- The contractor shall be responsible for obtaining any and all permits required by the Town of Bloomfield and/or the State of Connecticut for the work to be performed.

- The contractor shall provide, to the Common at Sinnott Farm, any documentation relating to insurance and/or workman’s compensation coverage of its employees.

- The contractor shall provide to the Common at Sinnott Farm their State of Connecticut contractors number, if applicable.

- The contractor shall provide to the Board of Directors of the Common at Sinnott Farm all pertinent information related to the materials to be used, if different than the specifications referenced above.

Please complete the attached sheets and forward them, at your earliest convenience, to the address shown above.

Thank you in advance for your cooperation.

The Board of Directors

The Common at Sinnott Farm

REQUEST FOR QUOTATION

THE COMMON AT SINNOTT FARM

# QUAIL RUN LANE

A.  Repair of pothole, catch basin, or areas of crazing.

## QUOTE:  ____________________

Work to include (please be specific):

B.  Crack repair.

QUOTE:  ___________________

Work to include (please be specific):

C.  Seal Coat.

QUOTE:  __________________

Work to include (please be specific):

D.  Contractor Recommendations.

QUOTE:  _________________

Work to include (please be specific):

Anticipated start date:  ______________

Anticipated Completion Date:  ______________

Note:  All price quotes referenced above shall include the cost of any and all permits required, and the coordination with any utility company(s) impacted by the work.

Company Name & Address (please print)

Submitted by (please print name and title)

## Signature of Submitter

## Date

REQUEST FOR QUOTATION

THE COMMON AT SINNOTT FARM

SPICE BUSH LANE

A.  Repair of pothole, catch basin, or areas of crazing.

## QUOTE:  ____________________

Work to include (please be specific):

B.  Crack repair.

QUOTE:  ___________________

Work to include (please be specific):

C.  Seal Coat.

QUOTE:  __________________

Work to include (please be specific):

D.  Contractor Recommendations.

QUOTE:  _________________

Work to include (please be specific):

Anticipated start date:  ______________

Anticipated Completion Date:  ______________

Note:  All price quotes referenced above shall include the cost of any and all permits required, and the coordination with any utility company(s) impacted by the work.

Company Name & Address (please print)

Submitted by (please print name and title)

## Signature of Submitter

## Date