FIRE SAFETY AND
EMERGENCY EVACUATION POLICY

Including campfire and woodland-fire procedures  |  2026–2027

Core rule  Life safety comes first. Raise the alarm, evacuate, account for everyone and call 999. No person should take a personal risk to fight a fire, recover property or search an unsafe area.

| Responsible Persons | Catherine Regi and Thomas Wild — Managing Partners |
| --- | --- |
| Day-to-day lead | The named session leader, acting as fire marshal for that session |
| Approved | 25 August 2026 |
| Review date | August 2027, or sooner following a fire, drill, near miss, site change or updated guidance |
| Applies to | All TWiG activities, children and young people, Associates, volunteers, parents and carers, visitors and contractors |
| Related documents | Fire Risk Assessment; Health and Safety Policy; Emergency Action Plan; Attendance Policy and register; First Aid Policy; Safeguarding Policy; Individual Risk Assessments and Personal Emergency Evacuation Plans; Incident and Accident procedures; Maintenance Log |

# 1. Purpose and scope

This policy explains how TWiG prevents fire, manages educational campfires and cooking fires, raises the alarm, evacuates the woodland and accounts for everyone. It applies to the woodland, pole barn, shelters, storage areas, access track, fire circle, equipment, temporary structures and any off-site activity for which TWiG has accepted responsibility.

Campfires can provide valuable opportunities for practical learning, regulation, cooperation and managed risk. They are never an entitlement or routine requirement. A planned fire proceeds only when the session leader is satisfied that it is necessary, appropriately staffed and safe in the current weather, vegetation, site and learner conditions.

# 2. Legal and guidance framework

TWiG manages its non-domestic premises in accordance with the Regulatory Reform (Fire Safety) Order 2005 and relevant amendments, including the strengthened recording and information duties introduced by section 156 of the Building Safety Act 2022. The Responsible Persons ensure that a suitable and sufficient written fire risk assessment, general fire precautions, emergency plan, information, instruction, training and maintenance arrangements are in place.

GOV.UK fire-safety guidance for people with legal duties and the five-step fire-risk-assessment checklist;

GOV.UK guidance for educational premises, open-air events and venues, small non-domestic premises and evacuation of disabled people, where relevant;

Norfolk County Council and Norfolk Fire and Rescue Service fire-safety advice;

Health and Safety at Work etc. Act 1974 and associated workplace requirements; and

Department for Education Non-school alternative provision voluntary national standards and Norfolk’s 2026 quality-assurance expectations.

Written assessment  The policy does not replace the site Fire Risk Assessment. The assessment identifies the actual hazards, people at risk, precautions, escape arrangements, equipment, actions, owners and review dates.

# 3. Responsibilities

## Managing Partners / Responsible Persons

ensure a suitable and sufficient fire risk assessment is completed by a competent person and kept under review;

cooperate and coordinate with any other person who has control of adjoining or shared premises;

provide and maintain appropriate alarm, escape, firefighting, first-aid and communication arrangements;

ensure emergency access, escape routes and assembly arrangements remain usable;

provide staff induction, annual refresher training and practical drills; and

review checks, drills, incidents, near misses and corrective actions.

## Session leader / fire marshal

checks conditions, staffing, attendance, escape routes, alarm and emergency equipment before the session;

decides whether any fire activity may take place and stops it if conditions change;

briefs adults and learners, allocates emergency roles and holds the current register or ensures it is immediately accessible;

takes command when the alarm is raised until responsibility passes to the emergency services; and

records relevant checks, drills, defects, incidents and follow-up actions.

## Associates, volunteers, parents, carers and visitors

All adults must follow instructions, maintain active supervision, keep exits clear, report hazards immediately and know the alarm and assembly arrangements. Parents who remain responsible for children during home-education sessions must bring them promptly to the assembly point and follow the session leader’s direction. Only TWiG adults may undertake allocated emergency roles; visitors are not relied upon to manage evacuation unless specifically briefed and competent.

# 4. Fire risk assessment and prevention

TWiG uses the five-step approach: identify fire hazards; identify people at risk; evaluate, remove or reduce risks; record findings, plan and train; and review. The assessment covers normal sessions, campfires, cooking, maintenance, contractors, lone working, events, mixed home-education/AP provision and foreseeable emergencies.

## Hazards considered

open flames, sparks, embers, hot cookware and ash;

dry grass, leaf litter, deadwood, brash, timber, shelters, fencing, stored fuels and overhanging vegetation;

electrical equipment, batteries, power banks, chargers, heating and cooking equipment;

smoking, arson, unauthorised access, hot work and contractor activity;

wind, heat, drought, wildfire warnings, restricted water supply and blocked or muddy escape routes; and

people who may need additional help to recognise an alarm, regulate, move quickly, follow a route or tolerate noise and crowds.

## General controls

Smoking, vaping near fire activities, fireworks, sky lanterns and unauthorised flames are prohibited.

Combustible waste and glass are removed; vegetation and stored materials are managed around ignition sources and escape routes.

Fuel is stored in limited quantities, away from ignition and public access. Flammable liquids are not used to light or accelerate a campfire.

Electrical items are visually checked before use and maintained in accordance with risk assessment and manufacturer guidance.

Emergency vehicle access, gates and escape routes are kept clear. Gates may be closed for site security but must not be locked or obstructed while the provision is operating.

Any defect affecting warning, escape or firefighting arrangements is acted on immediately; the affected area or activity is not used until safe.

# 5. When open fire is not permitted

The decision not to light is final  The session leader may cancel or extinguish a fire at any time. Curriculum plans are adapted without pressure to continue.

No campfire or other open flame is lit where any of the following applies:

a wildfire warning, Fire and Rescue Service restriction or landowner direction advises against fires;

prolonged dry weather, very dry vegetation, high temperature, drought or local wildfire activity makes spread foreseeable;

wind is strong, gusting, changeable or carries sparks or smoke towards people, vegetation, structures, neighbours or an escape route;

water and suitable extinguishing equipment are not immediately available;

the designated area, surrounding clearance or route is unsuitable or damaged;

staffing is insufficient for continuous competent supervision and learner support;

a learner’s current presentation, medical need or behaviour cannot be safely supported around fire; or

the session leader has any unresolved doubt about safe control or complete extinguishment.

The pre-session dynamic assessment considers current Met Office information, local Fire and Rescue Service advice and the actual woodland conditions. In hot, dry or windy periods, TWiG adopts a precautionary approach and uses no-flame alternatives.

# 6. Safe campfire procedure

## Designated area and equipment

Fire is permitted only in an established, risk-assessed fire area selected by TWiG.

The fire area and seating boundary are clearly defined and kept free of avoidable combustible material and trip hazards.

The fire is kept small and positioned as far as reasonably practicable from trees, overhanging vegetation, structures, stored fuel, vehicles and escape routes.

Water and the firefighting equipment identified by the Fire Risk Assessment are immediately available. A fire blanket and first-aid kit are accessible where cooking takes place.

A charged mobile phone or other reliable means of calling 999 is available.

## Lighting and supervision

A competent TWiG adult authorises, lights and remains responsible for the fire. A lit fire is never left unattended.

Fire steels or another agreed safe ignition method are used. Petrol, methylated spirit, aerosol, gel or other accelerants are prohibited.

Only clean, suitable untreated wood or approved fuel is burned. Plastics, rubbish, treated or painted timber and unknown materials are prohibited.

Long hair, loose clothing and dangling items are secured. The adult checks wind and smoke continuously.

Children enter the inner fire area only with permission and the agreed level of direct supervision. One learner at a time approaches unless the risk assessment states otherwise.

Fuel is placed—not thrown—from the side without reaching across the flame. Children do not move between the fire and seating boundary.

Cooking utensils are handled as hot until checked. Allergies, burns risk, food safety and cross-contamination are managed separately.

## Supporting children and young people

Instructions are adapted to age, communication, sensory, developmental and SEND needs. Adults model safe movement and rehearse the sequence away from the fire where helpful. A child may observe from a greater distance or decline the activity. Access is based on current readiness and individual risk assessment, not used as a reward or punishment.

## Smoke

Seating is positioned away from the smoke line and moved if wind changes. Anyone affected by smoke is moved to clean air immediately. Persistent coughing, breathing difficulty, asthma symptoms, eye irritation or distress is treated as a first-aid concern and escalated appropriately. The fire is extinguished if smoke cannot be controlled without exposing people or neighbours.

# 7. Extinguishing and closing the fire area

Stop adding fuel in sufficient time and allow material to burn down under supervision.

Separate remaining pieces safely, apply water gradually and stir the embers and ash.

Repeat wetting and stirring until there is no flame, smoke, steam, glow or detectable heat. Check the entire fire bed, not only the surface.

Do not bury hot material or leave it to extinguish unattended. Hot ash is never placed in a combustible container or dispersed in the woodland.

Keep the area supervised and inaccessible until cold. Record and report any concern about incomplete extinguishment.

Dispose of fully cold ash only through the method approved in the Fire Risk Assessment and site arrangements.

Cold means cold  A fire is not safe because visible flames have stopped. The responsible adult confirms that the fire bed and larger fuel remain cold before the area is left.

# 8. Alarm and communication

The emergency hand bell is the primary site-wide warning. The signal is continuous ringing accompanied by a clear shouted instruction of “FIRE—ASSEMBLY POINT”. Radios, mobile phones or repeated verbal calls are used as a backup, but no person should delay evacuation to collect equipment.

The bell is positioned so an adult can reach it promptly and can be heard across the occupied areas identified by the Fire Risk Assessment.

The session leader checks that it is present and unobstructed before opening. A functional test is recorded at the frequency set in Appendix B and during drills.

Because much of the provision is outdoors and continuously supervised, adults provide active fire detection. Any enclosed or partly enclosed occupied structure is assessed separately for suitable automatic detection, alarm, signage and emergency lighting.

On discovering fire or smoke, any person should alert the nearest adult immediately. An adult raises the alarm and calls 999 without delay where fire is uncontrolled, spreading, threatens vegetation or structures, or creates any doubt about safety.

# 9. Emergency evacuation procedure

Primary assembly point  The pole barn is the primary assembly point only when it and the route to it are safely away from the fire, smoke and wind direction. The secondary off-site assembly point is the concrete pad at the entrance to Meadow Harvest. The session leader may direct another safer upwind location if either point or route is compromised.

Raise the alarm. Shout “FIRE”, ring the emergency bell and make the situation known.

Call 999. Give the site address and location information from the Emergency Action Plan, describe access and send an adult to guide the Fire and Rescue Service if safe.

Evacuate immediately by the safest clear route. Do not stop for belongings, tools or equipment.

Adults take responsibility for their allocated learners. One adult leads and another sweeps only areas that can be checked without risk; nobody enters smoke or an unsafe structure.

Assemble at the safe point, keep groups together and take the register and headcount. Check children, Associates, volunteers, parents, visitors and contractors.

Report any missing person and their last known location immediately to the session leader and Fire and Rescue Service. Do not re-enter or carry out an unsafe search.

Provide first aid in a safe location and request an ambulance where required.

Remain at the assembly point, or move as directed, until the Fire and Rescue Service or session leader confirms what happens next. Nobody returns until authorised.

## Off-site movement to Meadow Harvest

Where evacuation beyond the woodland is required and the route is safe, a walking group is formed with a TWiG adult at the front and rear and additional adults alongside according to individual need. The group follows the safest available route and road-safety arrangements. A second register and headcount is completed on arrival. If smoke, fire, traffic or another hazard compromises this route, the session leader selects an alternative upwind exit or protected location and informs 999.

# 10. People who may need additional assistance

Fire planning considers children and adults who may have reduced mobility, sensory or communication differences, anxiety, trauma responses, medical needs, limited hazard awareness or difficulty following group instructions. Where a person may need individual arrangements, TWiG records them in an Individual Risk Assessment and, where appropriate, a Personal Emergency Evacuation Plan (PEEP).

a named adult and backup adult;

the clearest accessible route and an alternative route;

how the alarm and instruction will be communicated;

mobility, medical, sensory and regulation support or equipment;

how separation, freezing, hiding, running or refusal will be managed safely; and

how arrangements will be practised sensitively and reviewed with the person, family and commissioner.

A parent’s presence does not replace TWiG’s duty to plan for commissioned AP learners. During home-education sessions where a parent remains responsible for their child, TWiG still briefs the parent and includes both in site evacuation and accounting arrangements.

# 11. Use of firefighting equipment

The priority is alarm, evacuation and calling 999. Only an adult who is competent, has the correct equipment, has a clear escape route and can act without personal risk may attempt to extinguish a very small, contained fire. They must stop immediately if the fire grows, produces significant smoke, involves gas, electricity or an unknown substance, threatens vegetation or cuts off escape. Children and visitors must never fight a fire.

# 12. Training, induction and drills

Associates receive fire-safety induction before working independently and annual refresher training covering hazards, alarm, evacuation, 999 information, registers, equipment limits and individual assistance.

Adults who lead campfires must be competent in TWiG’s procedure, activity risk assessment, extinguishment, burns first aid and the decision to cancel.

Children are shown the alarm, boundaries and assembly arrangements during induction using accessible explanation, demonstration and rehearsal.

Each regular group practises evacuation at least termly and after significant changes. New learners are familiarised without waiting for the next drill.

Drills are trauma-informed: children are prepared where this is necessary, noise and uncertainty are adapted, and learning is achieved without avoidable distress.

The drill log records date, session, people present, alarm method, evacuation time, headcount, PEEP effectiveness, issues and completed actions.

# 13. Inspection, maintenance and records

The Responsible Persons maintain a fire-safety file or log containing the current Fire Risk Assessment, emergency plan, equipment checks and service information, alarm tests, drills, training, defects, maintenance, incidents, near misses and corrective actions. Records identify who completed the action and when.

## Review triggers

at least annual formal review and periodic dynamic review during operation;

a fire, false alarm, near miss, burn, smoke exposure or unsuccessful drill;

significant change to site layout, structures, access, equipment, activity, staffing or occupancy;

a new learner or adult whose needs affect evacuation;

severe weather, drought, storm damage, vegetation change or local wildfire event;

a defect in alarm, escape, emergency access or firefighting equipment; or

new legislation, Fire and Rescue Service advice, quality-assurance finding or commissioner concern.

# 14. Following a fire, evacuation or near miss

Provide first aid and emergency medical support; preserve life and safety before evidence or property.

Notify parents or carers and the commissioning school or local authority without delay where a child is affected or the planned provision changes.

Record facts, decisions, timings, injuries, people contacted and action taken. Complete safeguarding records where relevant.

Notify the insurer, enforcing authority, HSE under RIDDOR, Environmental Health or another body where the legal threshold or advice requires it.

Secure the area and do not resume use until a competent assessment confirms it is safe.

Debrief adults and support children in a developmentally and trauma-informed way; avoid blame.

Review the Fire Risk Assessment, emergency plan, PEEPs, training and equipment and record completed corrective action.

# Appendix A — Fire emergency action card

| Action | What to do |
| --- | --- |
| 1 Raise alarm | Shout FIRE; ring the emergency bell; give clear evacuation instructions. |
| 2 Call 999 | Ask for Fire and Rescue. Use the full site location and access details in the Emergency Action Plan. |
| 3 Evacuate | Use the safest upwind route. Do not collect belongings or enter smoke. |
| 4 Assemble | Pole barn only if safe; otherwise Meadow Harvest concrete pad or the safer point directed. |
| 5 Account | Use the live register. Count every child and adult, including visitors and parents. |
| 6 Report missing | Tell the session leader and Fire and Rescue Service. Do not re-enter. |
| 7 First aid | Treat in a safe place and request an ambulance if required. |
| 8 Wait | Do not return until authorised by Fire and Rescue Service or the session leader. |

# Appendix B — Minimum checking schedule

| Frequency | Check / record |
| --- | --- |
| Before each opening/session | Escape routes, gates, access, assembly areas, expected register, communications and visible hazards. Campfire conditions and emergency water before any fire. |
| Monthly | Functional emergency-bell test; visual check of fire blanket, extinguishing equipment, signs and emergency information; record defects and action. |
| Termly | Evacuation drill for each regular group; review PEEPs and lessons learned. |
| At manufacturer / competent-person frequency | Service or replace extinguishers, fire blankets, electrical and other fire-safety equipment as applicable. |
| At least annually | Formal Fire Risk Assessment and policy review; training refresher; emergency information and contact review. |
| Immediately after change or event | Review following a fire, near miss, failed drill, new hazard, site alteration, significant learner need or updated guidance. |

# Key references

Regulatory Reform (Fire Safety) Order 2005.

Building Safety Act 2022, section 156 fire-safety duties.

GOV.UK, Fire safety: guidance for those with legal duties; workplace fire-safety responsibilities; and Fire safety risk assessment: five-step checklist.

GOV.UK, Fire safety risk assessment: educational premises; open-air events and venues; and means of escape for disabled people.

Norfolk County Council, fire-safety requirements for educational premises and countryside fire-safety advice.

Department for Education, Non-school alternative provision: voluntary national standards.

Norfolk Review of Non-school Alternative Provision 2026 self-evaluation framework.