The Valvoline Company                        Date Prepared: 01/14/02

MSDS No: 505.0169702-008.013I

ND 30 12/1 QT

__________________________________________________________________________

1.   CHEMICAL PRODUCT AND COMPANY IDENTIFICATION

Material Identity

General or Generic ID: PETROLEUM BASED-LUBRICATING OIL

Company                             Telephone Numbers

The Valvoline Company               Emergency:     1-800-274-5263

P.O. Box 14000

Lexington, KY  40512                Information:   1-859-357-7206

__________________________________________________________________________

2.   COMPOSITION/INFORMATION ON INGREDIENTS

Ingredient(s)                                CAS Number    % (by volume)

-------------------------------------------- ------------- -------------

ALIPHATIC PETROLEUM DISTILLATES                 64742-65-0   94.0-100.0

ZINC C1-C14 ALKYLDITHIOPHOSPHATE                68649-42-3       0.7

__________________________________________________________________________

3.   HAZARDS IDENTIFICATION

Potential Health Effects

Eye

Unlikely to cause eye irritation or injury.

Skin

May cause mild skin irritation.  Prolonged or repeated contact may

dry and crack the skin.

Swallowing

Swallowing small amounts of this material during normal handling

is not likely to cause harmful effects.  Swallowing large amounts

may be harmful.

Inhalation

It is possible to breathe this material under certain conditions

of handling and use (for example, during heating, spraying, or

stirring).  Symptoms usually occur at air concentrations higher

than the recommended exposure limits (See Section 8).

Symptoms of Exposure

stomach or intestinal upset (nausea, vomiting, diarrhea),

irritation (nose, throat, airways), pain in the abdomen.

Target Organ Effects

No data

Developmental Information

No data

Cancer Information

Used motor oil has been shown to cause skin cancer in laboratory

animals continually exposed by repeated applications.  Avoid

prolonged or repeated skin contact.

Other Health Effects

No data

Primary Route(s) of Entry

No data

__________________________________________________________________________

4.   FIRST AID MEASURES

Eyes

If symptoms develop, move individual away from exposure and into

fresh air.  Flush eyes gently with water while holding eyelids

apart.  If symptoms persist or there is any visual difficulty,

seek medical attention.

Skin

Remove contaminated clothing.  Wash exposed area with soap and

water.  If symptoms persist, seek medical attention.  Launder

clothing before reuse.

Swallowing

Do not induce vomiting.  Give one glass of milk or water, and get

medical attention immediately.  If possible, do not leave victim

unattended.

Inhalation

If symptoms develop, immediately move individual away from

exposure and into fresh air.  Seek immediate medical attention;

keep person warm and quiet.  If person is not breathing, begin

artificial respiration.  If breathing is difficult, administer

oxygen.

Note to Physicians

Preexisting disorders of the following organs ( or organ systems)

may be aggravated by exposure to this material:  skin.

__________________________________________________________________________

5.   FIRE FIGHTING MEASURES

Flash Point

>  430.0     F (221.1     C) COC

Explosive Limit

No data

Autoignition Temperature

No data

Hazardous Products of Combustion

May form:  carbon dioxide and carbon monoxide, oxides of sulfur,

nitrogen and phosphorus, various hydrocarbons.

Fire and Explosion Hazards

Never use welding or cutting torch on or near drum (even empty)

because product (even just residue) can ignite explosively.  Dense

smoke may be generated while burning.

Extinguishing Media

regular foam, carbon dioxide, dry chemical.

Fire Fighting Instructions

Water or foam may cause frothing which can be violent and possibly

endanger the life of the firefighter.  Wear a self-contained

breathing apparatus with a full facepiece operated in the positive

pressure demand mode with appropriate turn-out gear and chemical

resistant personal protective equipment.  Refer to the personal

protective equipment section of this MSDS.

NFPA Rating

Health - 1, Flammability - 1, Reactivity - 0

__________________________________________________________________________

6.   ACCIDENTAL RELEASE MEASURES

Small Spill

Absorb liquid on vermiculite, floor absorbent or other absorbent

material.

Large Spill

Prevent run-off to sewers, streams or other bodies of water. If

run-off occurs, notify proper authorities as required, that a

spill has occured.  Persons not wearing protective equipment

should be excluded from area of spill until clean-up has been

completed. Stop spill at source, dike area of spill to prevent

spreading, pump liquid to salvage tank. Remaining liquid may be

taken up on sand, clay, earth, floor absorbent, or other absorbent

material and shoveled into containers.

__________________________________________________________________________

7.   HANDLING AND STORAGE

Handling

Containers of this material may be hazardous when emptied.  Since

emptied containers retain product residues (vapor, liquid, and/or

solid), all hazard precautions given in the data sheet must be

observed.

Storage

Not applicable

__________________________________________________________________________

8.   EXPOSURE CONTROLS/PERSONAL PROTECTION

Eye Protection

Not required under normal conditions of use.  However, if misting

or splashing conditions exist, then safety glasses or chemical

splash goggles are advised.

Skin Protection

Not normally required.  However, wear resistant gloves such as

nitrile rubber to prevent irritation which may result from

prolonged or repeated skin contact with product., Wear normal work

clothing covering arms and legs..

Respiratory Protections

Not required under normal conditions of use.  However, if oil

mists are generated above recommended PEL/TLV of 5 mg/m3, then a

NIOSH/MSHA approved respirator is advised in absence of proper

environmental control.  (See your industrial hygienist.)

Engineering Controls

Not required under normal conditions of use.  However, if unusual

operating conditions exist, then provide sufficient mechanical

(general and/or local exhaust) ventilation to maintain exposure

below PEL/TLV (s).

Exposure Guidelines

Component

----------

ALIPHATIC PETROLEUM DISTILLATES (64742-65-0)

OSHA VPEL 5.000 mg/m3 - TWA Oil Mist

ACGIH TLV 5.000 mg/m3 - TWA Oil Mist

ZINC C1-C14 ALKYLDITHIOPHOSPHATE (68649-42-3)

No exposure limits established

__________________________________________________________________________

9.   PHYSICAL AND CHEMICAL PROPERTIES

Boiling Point

(for product) 575.0 F (301.6 C) @ 760.00 mmHg

Vapor Pressure

Not applicable

Specific Vapor Density

<  1.000       @ AIR=1

Specific Gravity

.888 @ 77.00 F

Liquid Density

7.390 lbs/gal @ 77.00 F

.888 kg/l @ 25.00 C

Percent Volatiles (Including Water)

No data

Evaporation Rate

SLOWER THAN ETHYL ETHER

Appearance

No data

State

LIQUID

Physical Form

HOMOGENEOUS SOLUTION

Color

LIGHT AMBER

Odor

No data

pH

Not applicable

__________________________________________________________________________

10.  STABILITY AND REACTIVITY

Hazardous Polymerization

Product will not undergo hazardous polymerization.

Hazardous Decomposition

May form:  carbon dioxide and carbon monoxide, oxides of sulfur,

nitrogen and phosphorus, various hydrocarbons.

Chemical Stability

Stable.

Incompatibility

Avoid contact with:  strong oxidizing agents.

__________________________________________________________________________

11.  TOXICOLOGICAL INFORMATION

No data

__________________________________________________________________________

12.  ECOLOGICAL INFORMATION

No data

__________________________________________________________________________

13.  DISPOSAL CONSIDERATION

Waste Management Information

Dispose of in accordance with all applicable local, state and

federal regulations.

__________________________________________________________________________

14.  TRANSPORT INFORMATION

DOT Information - 49 CFR 172.101

DOT Description:

Not Regulated

Container/Mode:

CASES/SURFACE - NO EXCEPTIONS

NOS Component:

None

RQ (Reportable Quantity) - 49 CFR 172.101

Not applicable

__________________________________________________________________________

15.  REGULATORY INFORMATION

US Federal Regulations

TSCA (Toxic Substances Control Act) Status

TSCA (UNITED STATES) The intentional ingredients of this

product are listed.

CERCLA RQ - 40 CFR 302.4

None

SARA 302 Components - 40 CFR 355 Appendix A

None

Section 311/312 Hazard Class - 40 CFR 370.2

Immediate(X)   Delayed( )   Fire( )   Reactive( )   Sudden

Release of Pressure( )

SARA 313 Components - 40 CFR 372.65

Section 313 Component(s)                 CAS Number

---------------------------------------- -------------

ZINC C1-C14 ALKYLDITHIOPHOSPHATE            68649-42-3

International Regulations

Inventory Status

DSL (CANADA) The intentional ingredients of this product are

listed.

State and Local Regulations

California Proposition 65

None

__________________________________________________________________________

16.  OTHER INFORMATION

The information accumulated herein is believed to be accurate but

is not warranted to be whether originating with the company or

not. Recipients are advised to confirm in advance of need that the

information is current, applicable, and suitable to their

circumstances.

Last Page